FW5090:  Biophysics and Ecosystem Processes

Jul. 1, 2015

Jiquan Chen

Changliang Shao

# Environmental variables

- Definitions

- Dry bulb temperature (Ta, oC):  aspirated air temperature.

- Wet bulb temperature (Tw, oC):  the temperature attained when the moist air is brought adiabatically to saturation by evaporation of water in the moist air.

- Dew point temperature (Td, oC):  the temperature at which saturation will occur if the moist air is cooled at constant pressure.

- Absolute humidity (Ea, g.m-3):  the mass of water vapor per unit volume of air.

- Relative humidity (hr, %): the ratio pf water vapor of the moist air relative to the saturation vapor pressure at the same temperature and pressure.

- Saturation vapor pressure (es, kPa)

- Actual (partial) vapor pressure (ea, kPa):

- Saturation vapor density (Es, g.m-3):  the maximum Ea for a given air temperature and vapor pressure.

- Actual vapor density (Ea, gm-3):  the same as absolute air moisture.

- Vapor pressure deficit (or VPD, kPa): the difference between the saturation and actual vapor pressure at the same temperature and pressure.

- Some important relationships

Variables calculation process based on the already known measurements relative humidity (hr) and air temperature (Ta):

  - Relationship between vapor density to pressure

Where E is in kg.m-3, e is in Pa, and T is in K.

  - Conversion of temperatures

T(oC )  = T(K) -273.15

T(oC) = T(oF)*9/5 +32

  - VPD

  - Calculation of es from Ta (Tetens equation)

where Ta is in C.  Alternatively, Fritschen (1979) and Lowry (1977) used the following power functions:

  - Calculation of Es from Ta

  - Calculation of actual vapor density from h

  - Calculation of actual vapor pressure (ea).  This is the reversion of 2.5

  - Calculation of Td (this is the reversion of the Tentens equation in 2.4).  Note that parameters are slightly different because of the source (Fritschen 1979).

  - Calculation of T w is given in Eq. 3.11 by Campbell & Norman (1998) on Page 42.  By re-arrange the equation, T w can be calculated as:

Where is called the thermodynamic psychrometer constant.  It has a value of 6.66*10-4 with a slight temperature dependence.  Note Ea should be in Pa.

SAS code example:

data data1;

set rawdata;

E=2170*exp(1)/(ta + 273.15);

es_P=0.6118*exp(17.502*ta/(ta+240.97));

Es_D=10000*es_p/(4.62*(ta+273.15));

Ea_D=h*Es_D/100;

ea_P=4.62*Ea_D*(ta+273.15)/10000;

Td=237.3*log10(ea_P/0.61078)/(17.269-log10(ea_P/0.61078));

Tw=(ea_P+6.66*Ea_D*Ta/10000)/(es_P+6.66*Ea_D/10000);

proc print data=data1;

run;

Results example:

(1)

From this graph, we can see that Hr normally reach to the highest in the early morning at 6-7am, and lowest at approximate 3pm. One the contrary, VPD shows inverse trend. It is not surprising since VPD = es - ea,  es increases as temperature increases while ea showed almost neglectable variation comparesd with es, which makes VPD show the same trend asTa. On the other hand, Hr is negatively related to Ta. As graph showed below:

(2)

From this graph, we can see es has clear temporal variation since it is a positive function of Ta. However, es is the function of both Ta and Hr, therefore showed very low temporal variability. Similarly, the temporal variation of Es and Ea show the same trend.

(3)

The temporal variation of Td is higher than Tw. Td is positively related to ea,  while Tw is the function of  ea, es and Ta.